# Brenner Gradient Topology-based Autofocusing Framework for Fourier Ptychography Microscopy

## Abstract

Fourier Ptychographic Microscopy (FPM) provides high-resolution phase retrieval but is only reliable as its initial defocus estimation. When imaging unstained, transparent biological samples, traditional intensity contrast-maximizing autofocus routines fundamentally fail. Standard gradient metrics are deceived by the complex wave dynamics of phase objects. To break through this optical limitation, we propose a novel, object-aware autofocusing framework driven by a multi-scale topological profiler. Our method evaluates the geometric curvature of the gradient to mathematically isolate macroscopic Transport of Intensity Equation wave propagation from microscopic digital ringing artifacts. This decoupling allows the system to accurately classify the specimen's physical nature. Functioning as an computational pathfinder, the framework cleanly bypasses anomalies due to phase function and deterministically routes optical data to the correct optimization logic, ensuring universally robust, high-fidelity FPM reconstructions across diverse and highly complex sample types.

## Introduction

Fourier Ptychographic Microscopy (FPM) achieves high-resolution, wide-field imaging by capturing a sequence of low-resolution images under varying oblique illuminations and stitching them together in the Fourier domain [1]. Because the sample is often displaced from the ideal focal plane, estimating the exact defocus distance (Z0) is a mandatory computational step for accurate phase retrieval  [1,2]. If the assumed focal plane is incorrect, the spatial frequency constraints conflict during the alternating projections loop, leading to severe reconstruction artifacts and non-convergence.

While algorithmic solutions like Embedded Pupil Function Recovery (EPRY) exist to correct minor aberrations, they possess a narrow capture range and often stall in local minima if the initial defocus error is too large [3]. Consequently, a rapid, highly accurate computational autofocusing pre-step is mandatory.

Traditionally, autofocusing is achieved by digitally propagating the central brightfield image and identifying the plane that maximizes a high-frequency focus measure, notably the Brenner gradient. While maximizing contrast is highly effective for amplitude objects (e.g., stained histology slides), it is fundamentally flawed when applied to unstained, transparent biological samples. Recent studies have empirically shown that the optimal focal plane for pure phase objects corresponds to a contrast minimum rather than a maximum  [4–7]. Phase objects interact with defocus and finite apertures in non-linear ways, causing the Brenner gradient to invert, skew, or falsely peak.

Existing autofocus algorithms require a priori knowledge of the sample type to decide whether to search for a maximum or a minimum. When a universal rule is blindly applied, the system falls over three distinct gradient anomalies:

- The Phase Inversion: For a weak phase object, the focal plane exhibits no amplitude modulation. Contrast only emerges when the object is defocused, converting the true focal plane into a global minimum on the Brenner curve [5,8,9].

- The Amplitude-Phase Crosstalk (Asymmetry): Complex biological objects contain both amplitude absorption and phase delays. The interference between the symmetric amplitude transfer function and the anti-symmetric phase transfer function [10,11] heavily skews the contrast curve, breaking both minimum and maximum search rules.

- The Gibbs Anomaly (NA Clipping): Strong phase objects diffract light at steep angles. A finite objective NA abruptly clips these high spatial frequencies. This hard low-pass filtering forces invisible phase transitions to manifest as visible diffraction ringing exactly at the focal plane (Gibbs phenomenon). This artifact creates a sharp, false local maximum (a "W-shaped" curve) that routinely traps contrast-maximizing algorithms [12–14].

There currently exists no universal framework capable of autonomously reading a gradient curve, identifying the physical nature of the sample (amplitude, phase, or complex), bypassing optical artifacts, and deterministically locking onto the correct focal plane prior to ptychographic reconstruction.

The framework proposes that the true optical focus of any sample can be deterministically located by decoding the underlying optical topology of its defocus gradient curve, rather than relying on a basic ‘maximize contrast’ rule. By evaluating the curvature parameters across multiple scales, an algorithm can autonomously diagnose the physical nature of the sample and dynamically bypass gradient anomalies without a priori knowledge.

## Theory

- The Physics of the Brenner Gradient: The Brenner gradient measures the high-frequency contrast of an image by summing the squared differences between pixels separated by a small distance. For a 2D intensity image Ix,y at a specific defocus plane z, the metric is defined as:

| Bz=x,yIx+2,y,z-Ix,y,z2+x,yIx,y+2,z-Ix,y+2,z2 | (1) |
| --- | --- |

How Bz behaves as a function of defocus z depends entirely on whether the object modulates the amplitude of the light (absorption) or the phase of the light (refractive index variations).

- The Transport of Intensity Equation (TIE) Connection: To understand this behavior mathematically, we turn to the Transport of Intensity Equation, which links intensity variations along the optical axis (z) to the phase profile ϕx,y of the object. For a sample with uniform illumination (I=I0), this is,

| ∂I∂z∝-∇2ϕ | (3) |
| --- | --- |

Defocusing a phase object (z ≠0) mathematically converts its invisible phase gradients (∇2ϕ) into visible intensity variations.

To construct a universal autofocusing framework, we must first mathematically decouple macroscopic wave propagation (which dictates true contrast) from microscopic digital artifacts.

- Macroscopic Contrast: The macroscopic behavior of a gradient curve is governed by the TIE. Across a broad defocus sweep (e.g., ±50 μm), a pure phase object forms a macroscopic valley (concave-up) in the Brenner gradient, whereas an amplitude object forms a peak (concave-down).

- Microscopic Artifacts: The fatal flaw of the Brenner gradient occurs precisely at the centre of the phase valley (Z=0). According to continuous optical physics, a pure phase object requires infinite spatial bandwidth to maintain zero amplitude contrast at focus [15].However, a physical microscope utilizes an objective lens with a finite NA. When a strong phase object contains sharp phase transitions, it generates high spatial frequencies that exceed the NA's collection angle. The abrupt truncation of these frequencies by the lens acts as a low-pass filter, physically forcing the invisible phase boundaries to manifest as visible, high-contrast amplitude ringing exactly at the focal plane.

Figure 1 Brenner Gradient Topology for various object functions

## Methodology: The Object-Aware Autofocus Framework

To overcome these limitations, we propose a multi-scale topological profiler that autonomously classifies the physical nature of the specimen and routes the data to the correct optimization logic.

- The Rapid Profiler: Rather than performing a computationally expensive multi-LED reconstruction at every Z-plane, it utilizes only the central brightfield LED. The framework simulates a fast Z-sweep (scaled dynamically based on the objective's Depth of Field) and generates a normalized Brenner gradient curve.

- Multi-Scale Curvature Extraction: To recognise the Gibbs anomaly, the framework evaluates the Brenner curve at two distinct spatial scales, alongside a measure of global symmetry:

- Macro-Curvature (Cmacro): Evaluated at ≈5 × Depth of Field (DoF). This isolates the broad TIE physics.

- Micro-Curvature (Cmicro): Evaluated at ≈1 × DoF directly around Z=0. This detects the NA clipping artifacts.

- Brenner Asymmetry Index (BAI): Calculates the area imbalance between positive and negative defocus. A high BAI detects amplitude-phase crosstalk inherent to complex, absorbing tissues.

- Phase III: The Autonomous Logic Gate: The extracted indices are fed into a deterministic routing matrix:

- Pure Amplitude (Cmacro<0,Cmicro<τ): The algorithm safely locks onto the Global Maximum.

- Weak Phase (Cmacro>0,Cmicro≥τ): The algorithm correctly recognizes a true TIE valley and locks onto the Global Minimum.

- Corrupted / Strong Phase (Cmacro>0,Cmicro<τ): The algorithm detects the Gibbs anomaly (false peak). It rejects the central-LED curve to prevent an erroneous focus lock.

- Complex Object (BAI>τbai): The algorithm detects a highly skewed topology caused by absorption cross-talk.

Figure 2 Brenner Gradient based Topological Profiler

If a corrupted or complex object is detected, the framework triggers the fallback mechanism: Universal Artifact Minimization. The framework routes the system to a lightweight, Multi-LED FPM loop. Because an incorrect Z-guess forces the alternating projections algorithm to dump constraint errors into the reconstructed amplitude channel, the true focal plane is mathematically guaranteed to be the absolute minimum of the reconstructed high-resolution Brenner gradient.

## Discussion

It is crucial to distinguish this framework from existing solvers like EPRY. EPRY is a microscopic mechanism, highly effective at recovering sub-wavelength pupil aberrations, but inherently vulnerable to local minima if initialized at the wrong focal plane (a frequent occurrence if fed a false Gibbs peak). Furthermore, running EPRY blindly across a massive Z-sweep is computationally prohibitive for high-throughput healthcare applications.

This framework acts as the macroscopic pathfinder. By autonomously profiling the physics of the sample at a fraction of the computational cost (using a single LED), it deterministically bypasses the local minima traps caused by NA clipping and complex scattering. It delivers the dataset to the exact global focal plane, allowing algorithms like EPRY to be initialized safely and converge rapidly.

## Conclusion

Traditional autofocus algorithms fail on biological samples because they blindly apply a universal contrast-maximizing rule without recognizing the physical nature of the object. The proposed multi-scale topological framework overcomes this by autonomously profiling the macroscopic and microscopic physics of the Brenner curve. By acting as an macroscopic pathfinder, it bypasses the local minima traps caused by NA clipping and complex scattering. Ultimately, this framework deterministically delivers the dataset to the exact global focal plane, allowing high-resolution phase retrieval algorithms to initialize safely and converge rapidly.

References

1. 	"Wide-field, high-resolution Fourier ptychographic microscopy | Nature Photonics," https://www.nature.com/articles/nphoton.2013.187.

2. 	X. Ou, G. Zheng, and C. Yang, "Embedded pupil function recovery for Fourier ptychographic microscopy," Opt. Express 22, 4960 (2014).

3. 	M. Liu, Y. Wu, Z. Luo, J. Liang, R. Xie, J. Luo, and L. Yan, "Fast digital refocusing in Fourier ptychographic microscopy based on a multiscale fusion CNN," Optics & Laser Technology 184, 112554 (2025).

4. 	S. Schoell, F. Mualla, B. Sommerfeldt, S. Steidl, A. Maier, R. Buchholz, and J. Hornegger, "Influence of the phase effect on gradient-based and statistics-based focus measures in bright field microscopy," Journal of Microscopy 254, 65–74 (2014).

5. 	H. S. Park, S. Ceballos, W. J. Eldridge, and A. Wax, "Invited Article: Digital refocusing in quantitative phase imaging for flowing red blood cells," APL Photonics 3, 110802 (2018).

6. 	S. C. Shanmugavel, V. Senanayake, D. Suh, J. M. Gross, and S. Chowdhury, "Maskless and on-chip LED-array microscope with spatially varying angle calibration for centimeter-scale phase imaging," Optica, OPTICA 13, 247–258 (2026).

7. 	S. Zhang, G. Zhou, C. Zheng, T. Li, Y. Hu, and Q. Hao, "Fast digital refocusing and depth of field extended Fourier ptychography microscopy," Biomed. Opt. Express, BOE 12, 5544–5558 (2021).

8. 	J. F. Brenner, B. S. Dew, J. B. Horton, T. King, P. W. Neurath, and W. D. Selles, "An automated microscope for cytologic research a preliminary evaluation.," J Histochem Cytochem. 24, 100–111 (1976).

9. 	S. Schoell, F. Mualla, B. Sommerfeldt, S. Steidl, A. Maier, R. Buchholz, and J. Hornegger, "Influence of the phase effect on gradient‐based and statistics‐based focus measures in bright field microscopy," (n.d.).

10. 	C. J. R. Sheppard, "Defocused transfer function for a partially coherent microscope and application to phase retrieval," J. Opt. Soc. Am. A, JOSAA 21, 828–831 (2004).

11. 	C. Zuo, J. Sun, J. Li, J. Zhang, A. Asundi, and Q. Chen, "High-resolution transport-of-intensity quantitative phase microscopy with annular illumination," Sci Rep 7, 7654 (2017).

12. 	Y. Ichioka, K. Yamamoto, and T. Suzuki, "Defocused image of a periodic complex object in an optical system under partially coherent illumination," J. Opt. Soc. Am., JOSA 66, 932–938 (1976).

13. 	Y. Ichioka and T. Suzuki, "Image of a periodic complex object in an optical system under partially coherent illumination," J. Opt. Soc. Am. 66, 921 (1976).

14. 	F. Dubois, C. Schockaert, N. Callens, and C. Yourassowsky, "Focus plane detection criteria in digital holography microscopy by amplitude analysis," Opt. Express, OE 14, 5895–5908 (2006).

15. 	"Introduction to Fourier Optics 2ed Goodman J.W.pdf," https://docs.google.com/file/d/0B78A_rsP6RDSS3VRWk12Y2FUcVk/edit?usp=embed_facebook.