# Summary of Real-time multi-angle projection imaging of biological dynamics by Chang et. al. (2021)

Gaurang Garg

PhD student, H-ORG, CBME

# 1. Citation

Chang, BJ., Manton, J.D., Sapoznik, E. et al. Real-time multi-angle projection imaging of biological dynamics. Nat Methods 18, 829–834 (2021). https://doi.org/10.1038/s41592-021-01175-7

# 2. Summary of Contents

## 2.1. Paper Type

Novel integration technique to enhance performance on the conventional system for projection imaging of biological dynamics

## 2.2. Problem Description

Modern quantitative 3D imaging is often too slow to capture rapid biological dynamics because it is constrained by the limited rate of sequential acquisition of 2D focal planes. While current projection methods can increase speed, they typically deteriorate lateral resolution, require complex hardware, or are limited to a single viewing angle. Additionally, observing dense samples from multiple perspectives usually requires physical specimen rotation or multiple detection paths, both of which are impractical for live-cell imaging.

## 2.3. What is it?

The system operates on the principle of optical shearing, inspired by the shear-warp transform. By synchronizing a lateral sweep of galvanometric mirrors with the axial scan of the microscope's focal plane, the system can instantaneously integrate 3D information into a single 2D camera exposure equivalent to a projection of the rotated sample.

## 2.4. Study Design

- Hardware Setup: The researchers built a simple, custom scan unit consisting of two galvanometric mirrors mounted in a 3D-printed adapter to introduce a controlled lateral shift (shear) into the image during the axial scan. Synchronizing this shift with the z-scan allows the camera to integrate a 3D volume into a single 2D exposure.

- Microscope Integration: They integrated the unit into Spinning Disk Confocal (SDCM), Multi-direction Selective Plane (mSPIM), Oblique Plane (OPM), and Lattice Light-Sheet (LLSM) microscopes to demonstrate that the technology can augment any system with optical sectioning capability without requiring major hardware changes.

- Experimental Validation: They imaged the same cells (e.g., MV3 cancer cells) using both a traditional 334-slice Z-stack and a single-frame "Pro" acquisition to confirm if the two methods produced similar results.

- Biological Applications:

  - Versatility Testing on Denser Tissues and Subcellular Structures through imaging and tracking.

    - Vasculature of a zebrafish embryo

    - Spheroids of A375 cancer cells (at 0° and 45° viewing angles)

    - Colorectal cancer cells embedded in collagen

    - Subcellular dynamics like mitochondria and lysosomes in HeLa cells.

  - Intuitive sample exploration through projection imaging of cultured neurons to intuitively navigate structurally complex samples, providing instant 3D contextual cues in a single frame

  - High speed biological dynamics imaging to observe previously hidden dynamics

    - Tracked rapid formation/retraction of pressure-based blebs in MV3 cells

    - Captured a time series of calcium waves propagating through the dendritic arbors of cultured neurons at up to 119 Hz.

  - Localization: Triangulated the locations of nanoparticles in cancer cell using two views differing by 18° to prove that complete 3D volumes of sparse objects can be accurately localized and reconstructed using just two acquired projections instead of hundreds of slices.

  - By integrating a beam splitter and a second camera lacking the scan unit, they simultaneously acquired two orthogonal projections of a beating zebrafish heart at 10 Hz to bypass the motion blurring and stripe artifacts.

## 2.5. Results

The paper proved that the galvo-based projection of MV3 cells is just as accurate as imaged through traditional LLSM. Pro method reduces imaging time by over 100-fold and data overhead by over 300-fold without sacrificing image quality. The system revealed the entire neuron and its network of connections in a single view. High speed enabled observation of rapid processes such as distinct phases of bleb growth (a fast initial phase followed by a slower second phase), challenging the existing biological model of purely monotonic bleb expansion, recorded calcium waves propagating through the dendritic arbors of cultured neurons at up to 119 Hz and the high-resolution imaging of a beating zebrafish heart, while also supporting stereoscopic 3D visualization.

## 2.6. Conclusion

The study successfully demonstrates that the galvo-based scan unit can effectively convert conventional microscopes into high-speed projection systems with less data overhead without compromising optical quality. The technique is integrable across optical setups like mSPIM, OPM, LLSM and SDCM. The system maintains high spatial resolution without the need for sample rotation. The rapid acquisition of projected angles enabled advanced applications such as multi-viewing, VR viewing and localization.

# 3. Abstract

The researchers presented a modular, galvanometric scan unit that converts any camera-based microscope with optical sectioning into a high-speed, multi-angle projection imaging system. By leveraging the shear-warp transform through hardware-based optical shearing, the device integrates 3D volumetric information into a single 2D camera exposure. This approach accelerates acquisition rates by over 100-fold and reduces data overhead by up to 334-fold compared to traditional focal stacks. The technology is easily integrable and maintains high lateral resolution without requiring physical sample rotation or multiple detection paths. This enables imaging of multiple specimens like sub-cellular structures to vasculature of zebrafish embryo and observation of ultra-fast biological dynamics, such as 119 Hz calcium signalling in neurons and the high-resolution imaging of a beating zebrafish heart.

# 4. Novelty

The primary novelty of this work is the introduction of a modular, galvanometric scan unit that enables multi-angle projection imaging through hardware-based optical shearing. By exploiting the shear-warp transform, the system can instantaneously capture projections from arbitrary perspectives.

# 5. Limitations and Criticism

- Aberrations: Method is based on sum-intensity projection and low-contrast signal may get obscured and complex samples would provide enough sample aberrations to not recovered effectively.

- Camera: Shearing essentially stretches the image. To maintain a given field of view without clipping the peripheral regions of the image, a larger camera chip is required. Study of dynamic processes also is limited by acquisition rate of camera.

# 6. Your final thoughts

The scan unit is highly integrable for our setup, but it requires solving the problem that if shear transform is introduced at the low-resolution images, what would happen? We would have multiple single exposed low-resolution images from different angles and instead of developing high-resolution single thin slice images, we can reconstruct a high-resolution projection of a 3D volume.