# Summary of Video-rate imaging of biological dynamics at centimetre scale and micrometre resolution by Fan et al. (2019)

Gaurang Garg

PhD Scholar

# 1. Citation

Fan, J., Suo, J., Wu, J. et al. Video-rate imaging of biological dynamics at centimetre scale and micrometre resolution. Nat. Photonics 13, 809–816 (2019).

# 2. Summary of Contents

## 2.1. Paper Type

This paper presents a new technology and its application-based study. It details the optical design, computational architecture, and biological validation of a novel high-throughput imaging platform.

## 2.2. Problem Description

Conventional microscopes suffer from a fundamental trade-off between the achievable field of view (FOV) and spatial resolution, heavily restricted by the space-bandwidth product (SBP) theorem and scale-dependent geometric aberrations. Additionally, capturing large-scale biological dynamics at high speeds is bottlenecked by the immense data throughput required. Current techniques require slow step-and-repeat image stitching, or rely on Fourier ptychography which is too slow (e.g., ~3 minutes per gigapixel) for capturing fast, dynamic biological processes in vivo.

## 2.3. What is it?

The paper introduces the real-time, ultra-large-scale, high-resolution (RUSH) macroscope. It breaks the SBP bottleneck using a "flat-curved-flat" imaging strategy. Instead of forcing a massive flat image plane, which induces severe optical aberrations like field curvature at large scales; the custom objective magnifies the sample onto a concave intermediate spherical image surface. This surface is then divided and seamlessly relayed to parallel sCMOS cameras.

## 2.4. Study Design

- Optical Architecture: Primary step involved designing a custom objective lens capable of handling severe field curvature. The objective consists of 14 optical elements arranged in 11 groups. It was engineered to achieve a 10 mm x 12 mm field of view with a NA of 0.35 across a wavelength range of 420-680 nm. The setup projects a spherical intermediate image, which is then divided and relayed by a 5x7 relay lens array onto 35 individual sCMOS sensors. For the fluorescence modality, an LED light source was used with a 470 nm bandpass excitation filter and a 525 nm emission filter.

Figure 1 a, Diagram of the RUSH system. b, Illustration of the FOV of the RUSH system. Scale bar, 2,000 μm. c, Average FWHM of the 500-nm-diameter fluorescence beads across the whole FOV.

- System Calibration: Because the system relies on 35 distinct sub-fields with only a 3.6% overlap, the researchers implemented a fast computation-in-loop calibration method using spatiotemporal structured illumination, achieving a sub-pixel stitching precision of 0.2 pixels. To manage the massive data throughput of 5.1 gigapixels per second, the data pipeline utilized a 9-node computer cluster and a distributed file system spread across 324 hard disks.

- Optical Benchmarking: The physical optics were rigorously characterized by measuring the optical resolution using 500-nm-diameter fluorescent beads. To confirm spatial uniformity, they divided each sub-FOV into smaller blocks and calculated the average full-width at half-maximum (FWHM) of the point spread function (PSF) in each region.

- Biological Imaging:

  - In Vitro Cellular Dynamics: To validate the system's ability to capture high-speed, large-scale network activity, the team performed calcium imaging on cultured cells. They isolated and cultured rat primary cardiomyocytes loaded with Fluo-4 AM, and primary cortical neurons loaded with Fluo-8 AM.

  - Ex Vivo Human Tissue Studies: To demonstrate clinical relevance, the study design incorporated acute human cortical slices obtained from epilepsy patients. The slices were loaded with OGB1-AM calcium indicator. The protocol included observing both spontaneous calcium waves and evoked recurrent network activity triggered by a bipolar electrode delivering electrical shocks.

  - In Vivo Whole-Brain Functional Imaging: The final phase of the study tested the macroscope in living, behaving mammals. Craniotomies were performed to install optical windows on mice to track the real-time migration of immune cells and chronic imaging of neurovascular coupling

## 2.5. Results

The RUSH platform successfully achieved a 10 mm x 12 mm FOV with a uniform lateral resolution of 1.20 µm (after deconvolution). Operating the 35 sCMOS cameras in parallel yielded a massive data throughput of 5.1 gigapixels per second. Biologically, the system successfully tracked the propagation of epileptiform activity, revealing inconsistent wave directions and wave-splitting. In vivo, it successfully tracked individual immune cells migrating across the entire superficial cortex at average speeds of 65 µm/s and captured the temporal delay (~1.36 s) between neural calcium spikes and subsequent vascular dilation.

## 2.6. Conclusion

By utilizing a flat-curved-flat optical cascade, the RUSH macroscope successfully breaks the space-bandwidth product bottleneck. It enables, for the first time, video-rate, gigapixel imaging of biological dynamics at the centimetre scale while maintaining sub-cellular resolution, paving the way for advanced studies in systems biology and immunology.

# 3. Abstract

Systems biology relies heavily on understanding how localized cellular events cascade into macroscopic physiological behaviours. However, modern optical microscopy is severely limited by scale-dependent geometric aberrations, forcing researchers to choose between a large field of view (FOV) or high spatial resolution. This paper introduces the RUSH macroscope, an imaging platform that circumvents this bottleneck by employing a flat-curved-flat optical relay. By projecting a 10 mm x 12 mm object plane onto a spherical intermediate surface, the system actively mitigates field curvature. This surface is then split across 35 parallel sCMOS sensors, achieving a uniform 1.2 µm resolution at a video rate of 30 frames per second. RUSH delivers a staggering 5.1 gigapixels per second of data throughput, successfully demonstrated through the tracking of epileptiform waves in human brain tissue and brain-wide neurovascular coupling in awake mice.

# 4. Novelty

The primary novelty lies in translating astronomical telescope design principles (using curved image surfaces) into a microscopy framework. The integration of 14 custom optical elements to deliberately create a concave intermediate image, combined with a computational-in-loop calibration algorithm that allows 35 separate cameras to stitch images together with 0.2-pixel precision; is a major engineering breakthrough.

# 5. Limitations and Criticism

- Depth and Scattering: The primary modality of the RUSH system is wide-field fluorescence. Wide-field imaging is highly susceptible to tissue scattering, heavily restricting depth penetration compared to multiphoton or confocal microscopy.

- Data Bottleneck: Generating 5.1 gigapixels per second poses severe data storage and processing challenges. The researchers required a dedicated 9-node computer cluster and 324 hard disks (4 TB each) just to handle the raw data flow.

- Complexity and Accessibility: The necessity of 35 precisely aligned, water-cooled sCMOS cameras and highly specialized custom optics makes the RUSH system cost-prohibitive and difficult to adopt.

# 6. Your final thoughts

Instead of correcting curvature and trying to achieve planar flat image, they projected a curved intermediate image and placed the centre of that sphere at the exit pupil to achieve uniform resolution across a massive field. While RUSH uses complex hardware to capture a massive flat field, our takeaway should be computational. If we combine their parallel-acquisition architecture with our concepts in FPM and multislice algorithms, we could build a system that captures large-scale, 3D biological dynamics without the need for a 14-element custom objective. If we allow for a curved intermediate plane and instead correct the geometry computationally (or via a simpler split-sensor relay), we might achieve larger FOVs with fewer complex, expensive lenses.