Review Meeting- 052

Gaurang Garg

BioPhotonica Lab, Centre for Biomedical Engineering, IIT Delhi

2024-10-01 (Tuesday)

Title: Centimeter scale 3D imaging via multislice approach

Abstract: High throughput 3D Imaging for various applications requires higher value of SBP as wide FOV with high resolution. Lens-less maskless on-chip imaging framework has been able to achieve centimeter scale reconstruction with good resolution. 3D Imaging can be extended to the framework by modelling the sample based on multi-slice approach and implementing light-field refocusing as the guess estimate. Proposed theory could enable simple, fast 3D imaging with HR.

# 1. Introduction

The inherent trade-off between FOV and resolution as quantified through space-bandwidth product can be resolved through computational framework involving minimal hardware complexities; even as to achieve centimeter scale FOV (2.7 × 1.7 cm2) with a resolution of 1.38 μm equivalent to approx. 2.4 gigapixels phase images of transparent biological specimens [1]. The samples under consideration are thin on the ranges of few micrometers under approximations of 2D modelling  [2], see Supplementary. For various applications and studies, 2D data would not suffice and the need is to image in three dimensions; reconstructing volumetric data.

High resolution 3D imaging is possible with existing technologies but is limited to slow point scanning; as in the case for confocal microscopy and multi-photon microscopy. Wide-field tomography approaches for 3D imaging require multiple measurements at each angle for 2D phase retrieved images for reconstruction and angle scanning to capture the projections. Further, diffraction tomography is used where wave-optical effects are significant [3]. The physical optical setup for all these methods is somewhat complex and requires alignment and calibration beforehand. Method based on computational and mathematical framework with only one intensity image for each angle as opposed to multiple images to retrieve phase has been successful in achieving high resolution 3D imaging by exploiting angular diversity and asymmetry in pupil plane. Iterative Fourier Ptychography algorithm on a light field refocusing based guess solution has been able to achieve high SBP 3D data [3].

Both techniques bypass the fundamental SBP limit through computational approaches with two unique paradigms but also limited by their set of problems:

- High-Resolution 3D Volumetric Reconstruction: Multi-slice Fourier ptychography reconstructs 3D intensity and phase by capturing angular diversity and feeding the data into a wave-propagation model that accounts for multiple scattering. However, the setup remains constrained to an objective lens, fundamentally restricting the FOV to the millimeter scale at best.

- Centimeter scale on-chip 2D Phase Imaging: Lens-less on-chip technique entirely removes the objective lens, placing the sample directly above a bare image sensor to achieve centimeter-scale FOVs. Yet, computational framework models the sample as a single 2D transmission plane, suffering severe defocus artifacts if the biological tissue exceeds the system's narrow depth of field.

Furthermore, when multislice approach was validated using simulations and experiments, it showed significant deviation on encountering defocus diversity, [Figure 1], where defocus refers to the relative distance of test target from physical focal plane. Light field based refocusing showed degradation of lateral resolution due to diffraction effects as predicted by theory but for multi-slice Fourier Ptychography based reconstruction, the experimental results showed degradation of resolution as opposed to predicted, no degradation. This has been explained by LED position miscalibration as angular error becomes significant at higher defocus [3].

Figure 1 Comparison between simulation and experiment for the multislice approach and guess estimate using light field refocus  [3]

Centimeter scale framework includes calibration and precise correction of residual angular errors on the initial estimates through joint-optimization procedure which iteratively updates both the sample object function and spatially varying illumination angles [1,2].

Uniting these paradigms requires a new forward model that abandons the objective lens's pupil function entirely while expanding the lens-less framework from a single 2D sheet into a 3D scattering volume, while correcting for the severe geometric distortions inherent to illuminating massive FOVs.

# 2. Forward Model

The mathematical model simulates how each LED’s light would interact with the sample object and the intensity recorded by the sensor.

## Spatially Varying Illumination Field

In conventional objective-based ptychography, an LED array placed sufficiently far away is assumed to provide spatially coherent, uniform plane-wave illumination across the entire microscopic FOV. At the centimeter scale of a bare sensor, this assumption breaks down. LEDs emit spherical wavefronts, not planar ones. Therefore, a single LED strikes the center of the sample at a near-normal incidence but hits the periphery at highly oblique angles. A single LED illuminate different regions of the sample at varying angles.

To account for this, whole wide sensor area (FOV) must be discretized into a grid of smaller sub-regions within which the illumination from LED can be considered as uniform plane wave.

The subregion size is decided based on the illumination’s spatial coherence area at the sensor plane. The local area of the sub-region must be smaller than coherence area Ac given by van Cittert-Zernike theorem [4–6],

| Ac = (λ∙z/r)2 | (1) |
| --- | --- |

where z is the distance between LED array and the sample, r being the dimension of the LED of wavelength λ, see Supplementary.

Each lth LED can be thought of as extending monochromatic planar illumination to the nth sub-region incident at an angle θn(l). Thus, the incident planar electric field, can be mathematically represented as

| fi(l,n)(r) = exp (i2πvn∙r) | (2) |
| --- | --- |

Where vn are the spatial frequencies related to the tilted illumination angle by vn = (sinθx,n(l)λ , sinθy,n(l)λ) and r denotes the lateral coordinates.

## 3D Multi-slice Scattering Interaction

Extension to 3D imaging requires moving from 2D thin sample interaction to 3D volumetric scattering. We adopt multi-slice approximation where the object is modelled as comprising of multiple slices and the light interacts with each slice consecutively. Since we model the wavefront being modulated at each slice, the approximation does not require any weak or single-scattering assumption [3].

The object volume within the nth subregion is sliced into M discrete layers separated by a uniform medium of thickness δz. Each slice m is parameterized by an unknown 3D complex transmittance function On,mr.

The field propagates sequentially through the volume. At each slice, the light is first modulated by the complex transmittance, and the resulting exit wave is then propagated to the next slice using the angular spectrum method. The field exiting the slice m following the interaction with the complex transmittance can be represented as,

| gm(l,n)(r) = On,mr∙fm(l,n)(r) | (3) |
| --- | --- |

The exit-field then propagates to the next slice through a medium over inter-slice distance δz and is given by

|  | (4) |
| --- | --- |

where Hδz{⋅} denotes free-space wave propagation and is given by  [7,8]

| Hδz{⋅} = F-1F∙∙exp -i2πδz1λ2 - v2 | (5) |
| --- | --- |

## Lens-less Boundary Condition

We deviate from conventional ptychographic methods by not using the objective lens to achieve maximal frequency information by letting go of the low pass filtering. The unified on-chip framework expects that after passing through multiple slices through the 3D volume m = 1,2,3,…,M, the wavefront exiting the last slice would further propagate through free space over a distance zsample before reaching the sensor. The field reaching the active sensor area is given by

| fsensor(l,n)r=Hzsample gM(l,n)(r) | (6) |
| --- | --- |

The field would be detected as intensity fsensor(l,n)r2 and is continuous and carries all the information of the object function encoded in it. The practical limitations imposed by the sensor would require two modifications to the mathematical model:

- The sensor has a finite pixel pitch and would offer constraint on the resolution by imposing an achievable resolution limit. To achieve resolution beyond, and break sampling limit, the model must be able to recover a high-resolution grid that is downsampled to match physical dimensions of the sensor.

- The sensor has a physical measurement window (FOV), and the field must be cropped while being detected.

An operator Cσ with a downsampling factor σ accounting for both limitations would act on the calculated continuous, large bandwidth signal to give close prediction on the captured discrete, bandlimited intensity, which is:

| In(l)r = Cσfsensor(l,n)r2 | (7) |
| --- | --- |

Figure 2 Complete schematic of Forward Model

The forward model for the nth subregion being illuminated by  lth LED to form an image given by intensity distribution In(l)r, is as follows:

| In(l)r = CσHzsample On,Mr∙HδzOn,M-1r∙.....HδzOn,1r∙exp (i2πλsin(θn(l))∙r)2 | (8) |
| --- | --- |

# Hardware and System Calibration

The section comprises of the hardware design and core geometry based initial calculations and estimate required for the algorithm to work. To achieve centimeter-scale wide-field 3D imaging, the system architecture entirely discards the conventional objective lens, relying instead on a highly parameterized lensless on-chip geometry.

## Experimental Setup

The physical apparatus is designed to capture high space-bandwidth product (SBP) scattering data without mechanical scanning. The sample is placed directly above a bare, large FOV monochrome level CMOS sensor. A physical air gap is maintained between the sample and the active pixel array to allow for forward scattering and diffraction. Illumination is provided by a programmable 32 ×32 LED array  [1,3,9] (e.g., Adafruit Inc. with 4 mm element spacing), positioned approximately 30 cm above the sample to achieve spatial coherence. A microcontroller synchronizes the sequential activation of individual LEDs with the camera's global shutter via a hardware trigger pulse. To ensure quasi-monochromatic illumination with sufficient temporal coherence, a 10 nm FWHM bandpass filter is placed directly above the specimen [1] (20 nm bandwidth and 7.4 cm distance between the sample and LED array, in  [3]).

Figure 3 Schematic of On-chip cm scale imaging with LED array for multi-angled illumination

## Optical Alignment

Because the algorithm relies on shifting phase functions in Fourier space, defining a true optical axis is strictly necessary. After making LED array and image sensor coplanar through leveling tool, we establish alignment by partitioning the massive FOV into subregions and selecting a ‘reference’ subregion exactly at the center of the sensor.  This central subregion is physically aligned to be completely on axis with a designated ‘reference’ LED at the center of the array. We verify this orthogonality by capturing diffraction patterns of a symmetric calibration target (such as a Siemens star) at multiple defocus planes. When the reference LED and the central subregion are perfectly aligned, translating the sample axially induces zero lateral displacement in the resulting diffraction pattern, see Supplementary  [1,7,10].

Figure 4 Defocus correlation with lateral shift in diffraction patterns (a) Full FOV diffraction pattern of a phase target illuminated by a reference LED. A Siemens star pattern at the center (subregion A1) is used as the reference subregion for alignment. (c) Diffraction patterns from subregions before and after 1 mm axial displacement.  [1]

## Sample-to-sensor distance for 3D volumes

In 2D lensless imaging, the gap distance between the sample and sensor ($\Delta z$) is found by digitally back-propagating the central subregion's on-axis hologram and finding the depth that minimizes the Brenner gradient of the amplitude.

# Supplementary

## Thin Sample Approximation

The consideration of a sample whether it is thin and can be considered 2D is based on optical system’s Depth of Field (DOF). The 2D approximation holds if the thickness of the sample is less than or approximately equal to system’s DOF. Under this assumed region, the light rays do not significantly diffract or spread laterally while propagating through the sample. Within these bounds, entire physical volume can be modelled as a compressed infinitely thin 2D complex transmittance plane. This at plane of focus is equivalent to pure phase transmittance with no amplitude modulation. Amplitude fluctuations only arise due to defocus after the light exits the sample and propagates through the air gap to the sensor.

In the on-chip framework  [1,2], the researchers successfully utilized the 2D approximation to reconstruct 10 µm diameter microspheres , 12 µm thick zebrafish sections , and 16 µm thick mouse kidney sections.

Out of bounds of DOF, the 2D approximation physically breaks down. In thick samples, the light rays from different illumination angles take different paths across the sample. The light internally gets diffracted internally before exiting the sample. Because of path-dependency, angled data represents tomographic information. For accurate reconstruction the sample must be modelled as to account for internal scattering and diffraction effects, such as the multi-slice model, which splits the volume into a series of N thin slices. The wave field propagates from slice to slice, with each layer modulating the field sequentially.

In the multi-slice Fourier Ptychography framework  [3], the researchers imaged an algae sample with a total thickness of 100 µm. They computationally sectioned it into 11 slices separated by 10 µm each to accurately recover the 3D intensity and phase.

## Depth of Field

To know the physical range of 2D approximation and allowed sample thickness, we must calculate system’s axial resolution. For a lens-less system like the on-chip framework, the effective Numerical Aperture and thus the axial resolution is determined by the gap between the sample and sensor and physical parameters of the sensor. The geometric Numerical Aperture (NA) is dictated by how much of the scattered light the sensor can physically intercept. It is dependent on the maximum angle subtended between the sample and the edges of the sensor, which is equivalent to light gathering in conventional lens-based system [1,2].

| NA = n∙sinφ = W2∆z2+ W24 |  |
| --- | --- |

Where n is the refractive index of medium between the sample and sensor and W is the sensor width.

Figure 5 Effective NA of the on-chip system  [2]

Given W=7.4 mm and Δz=1 mm, and medium being air (n = 1) the theoretical effective NA = 0.94. Using standard formulas and wavelength (λ = 632nm), the DOF of the system comes out as,

| DOF ≈ λNA2 ≈ 0.71 μm |  |
| --- | --- |

Thus, any sample thicker than this value must be considered 3D volume. But, the authors reconstructed samples in the ranges that are not sub-micron successfully using 2D approximations [1] (10 µm diameter microspheres , 12 µm thick zebrafish sections , and 16 µm thick mouse kidney sections).

The system’s practical DOF is constrained by camera’s pixel pitch. The pixel size acts as a low-pass filter, limiting the practical resolution. According to the Nyquist-Shannon sampling theorem, the smallest feature you can resolve is twice the pixel pitch  [7,8]. For the pixel pitch as 1.85 μm, the smallest resolvable feature is 3.7 μm. The practical limited NA corresponding to this physical resolution is,

| NApractical = λresolution ≈ 0.17 |  |
| --- | --- |

The practical DOF can be calculated using this NA comes out as 21.8 μm. If the sample is in ranges of this limit, it can be approximated as thin.

## Coherence Area

The physical principle determining the spatial coherence area of an extended, quasi-monochromatic light source, as an LED, is governed by the van Cittert-Zernike theorem. The theorem establishes a relationship between the far-field diffraction generated due to specifically represented incoherent source of light in a region of space and the coherence resulting from the same source in the same region [4,5]. LEDs are a thermal incoherent light source and on propagating a certain distance it acquires localized coherence, the representation being 2D square LEDs. The farther the light travels from the LED, the larger the transverse area over which the phase fluctuations become correlated. The physical region across which these wavefronts maintain a predictable phase relationship is defined as the coherence area. According to the theorem, the spatial coherence area (Ac) of an extended source is inversely proportional to the solid angle (∆Ω) that the source subtends at the sensor plane.

| Ac ≈ λ2∆Ω |  |
| --- | --- |

For an LED with a physical emitting area As positioned at a distance z from the sample, the solid angle is approximated by ΔΩ≈As/z2. Substituting this,

| Ac≈λ2z2As |  |
| --- | --- |

For the two studies, the LED with an emitting area of 120 μm × 120 μm and wavelength of 632 nm, for a sample placed 30 cm [1,2] and 74 mm [3] above the LED array, the coherence area comes out to be 400 μm × 400 μm and 1580 μm × 1580 μm respectively. Since these patches are drastically smaller than a centimetre-scale sensor, the entire FOV is not spatially coherent. Computational framework must subdivide the massive FOV into smaller grids; within any subregion smaller than coherence area, the illumination can be mathematically approximated as a single coherent plane wave.

## Wave-Optic mechanics of Defocus Induced translation

To rigidly align the computational framework, the central reference subregion must receive perfectly orthogonal illumination. This is experimentally verified by axially displacing the sample (defocusing) and observing no lateral shift in the diffraction pattern. Conversely, off-axis LEDs produce diffraction patterns that translate laterally across the sensor as a function of defocus.

Let the 2D complex transmittance of the sample (e.g., the calibration target) be Or, where r=x,y. When illuminated by a plane wave from an off-axis LED with angle θx (assuming a 1D tilt), the incident wave has a specific spatial frequency u0:

| u0=sinθxλ |  |
| --- | --- |

The exit wave immediately after passing through the sample is

| gr=Orexpi2πu0x |  |
| --- | --- |

According to the shift property of the Fourier transform, multiplying by a complex exponential in real space results in a direct translation in Fourier space. Therefore, the frequency spectrum of the exit wave is:

| Gu=F{gr}=Ou-u0 |  |
| --- | --- |

where Ou is the Fourier transform of the un-tilted sample. The entire spectrum is shifted by u0. To propagate this exit wave across the sample-to-sensor gap (Δz), we multiply its spectrum by the free-space transfer function Hu. For the sake of exposing the underlying symmetry, we will use the paraxial (Fresnel) approximation of the transfer function [7],

| Hu≈exp-iπλΔzu2 |  |
| --- | --- |

When we propagate our tilted exit wave, the spectrum becomes:

| GΔzu=Ou-u0exp-iπλΔzu2 |  |
| --- | --- |

To understand how this defocus affects the shifted spectrum, we change our variable to center on the shifted spectrum [7]. Let u'=u-u0, which means u=u'+u0. Substituting this into the quadratic phase term of the defocus transfer function,

| -iπλΔzu2=-iπλΔzu'2+2u'u0+u02 |  |
| --- | --- |

Now, distributing this into the propagated spectrum we get,

| GΔzu'=Ou'⋅exp-iπλΔzu'2⋅exp-i2πλΔzu0u'⋅exp-iπλΔzu02 |  |
| --- | --- |

This equation is the key to the entire phenomenon. It contains three distinct phase terms acting on the centered spectrum Ou':

- exp-iπλΔzu'2: This is the standard quadratic defocus blur. It is the same blur the sample would experience if it were illuminated on axis.

- exp-iπλΔzu02: This is a global, constant phase offset due to the incident light. Since image sensors only detect intensity (magnitude squared), this term physically vanishes at the sensor plane.

- exp-i2πλΔzu0u': This is a linear phase ramp in Fourier space.

The Fourier Shift Theorem dictates that multiplying a frequency spectrum by a linear phase ramp exp-i2πΔxu is mathematically identical to shifting the object in real space by Δx

| F-1Ouexp-i2πΔxu=Ox-Δx, y |  |
| --- | --- |

By comparing our linear phase terms we get the physical translation distance,

| Δx=λΔzu0 |  |
| --- | --- |

Recall our initial definition of the spatial frequency u0=sinθx/λ. Substituting this back in yields,

| Δx=λΔzsinθxλ=Δzsinθx |  |
| --- | --- |

Applying a quadratic defocus phase to a shifted frequency spectrum naturally spawns a linear phase ramp. Therefore, defocusing a pattern under off-axis illumination is strictly and mathematically equivalent to the Fourier Shift Theorem. This dictates our hardware alignment protocol. If the central reference LED is perfectly on axis (θx=0, thus u0=0), the linear phase ramp exp-i2πλΔzu0u' becomes 1. No linear phase ramp exists, meaning changing the defocus Δz induces zero lateral translation. Any observed lateral shift of the central diffraction pattern during the physical z-sweep indicates a non-zero u0, proving the hardware is misaligned and requires calibration before the computational model can converge.  [7,10,11]