# Summary of 3D intensity and phase imaging from light field measurements in an LED array microscope by Tian et. al. [2015]

Gaurang Garg

PhD Scholar, CBME

# 1. Citation

Lei Tian and Laura Waller, "3D intensity and phase imaging from light field measurements in an LED array microscope," Optica 2, 104-111 (2015)

# 2. Summary of Contents

## 2.1. Paper Type

Paper presents an effective framework for 3D intensity and phase imaging.

## 2.2. Problem Description

Biomedical applications require 3D high-resolution imaging of specimens.

- Achieving high-speed acquisition alongside HR across large volumes remains a challenge.

- Conventional HR methods, such as multi-photon or confocal microscopy, are limited by slow point scanning through the 3D volume, while wide-field tomographic approaches require multiple measurements at each angle.

- Standard light field digital refocusing suffers from poor lateral resolution at smaller feature sizes because it relies on geometric optics and fails to account for prominent wave-optical diffraction effects.

## 2.3. What is it?

The paper presents a novel mathematical framework that recovers HR 3D intensity and phase from 4D light field data using Fourier ptychography. An iterative multi-slice reconstruction model is applied to multi-angled intensity images to estimate the complex transmittance of the sample, correcting for diffraction artifacts and multiple scattering effects. This combined approach enables gigavoxel-scale 3D imaging with resolution beyond the diffraction limit, while maintaining a large FOV.

## 2.4. Study Design

- Data Acquisition and Initial Estimation: LED array based multi-angled intensity images are captured to encode 3D information through angular diversity. The intensity images are first processed using standard light field digital refocusing algorithms. This gives an initial 3D intensity map by mathematically undoing the geometric shifts that occur when light rays propagate from off-axis illumination sources. This geometric approximation provides a highly constrained, close initial guess of energy distribution for the complex 3D structure of the sample. Starting with this close approximation prevents the iterative algorithm from getting trapped in local minima and ensures convergence.

Figure 1 (1) LED array based microscope setup; (2) 4D space-angle data for light field focusing by calculating the geometric shifts for each angle of illumination; (3) (a) raw images collected from varying illumination angles θx for a two-slice sample (b) Two space-angle (x − θx) plots, where features from different depths create tilted lines with depth-dependent slopes, surrounded by diffraction fringes.

- Forward Modelling: The algorithm models the thick sample as a discrete series of thin slices separated by a uniform medium, such as air and calculates the wave field propagating through the sample using a sequence of multiply-and-propagate operations, where each slice physically modulates the incident light field. This accounts for the complex wave-optical effects that geometric optics miss. Since the approach makes no weak or single-scattering approximations, it allows the system to accurately model and eventually correct for multiple scattering events as light travels through a dense volume.

- Reconstruction Algorithm: An iterative gradient descent-based optimization algorithm is applied to the multislice model. For each illumination angle, the algorithm updates the complex transmittance function through back-propagation and complex pupil function of the lens by minimizing the difference between the actual measured intensity and the prediction by the model. This step functions as a nonlinear 3D deconvolution that computationally removes out-of-plane blur and removes optical aberrations inherent to the physical system. By stitching together different sections of Fourier space captured by the varied LED angles, it synthesizes a larger NA.

- Validation Experiments:

  - Stacked Resolution Targets: Two resolution targets were placed at different depths to experimentally characterize lateral resolution at multiple planes and verify that out-of-plane blur was successfully removed compared to physical focusing.

  - Thick Biological Sample: A continuous Spirogyra algae sample was imaged to demonstrate the method's capability to recover both absorption and phase effects in a complex, naturally scattering 3D volume.

## 2.5. Results

A short summary of the important results discussed and realised in the paper.

## 2.6. Conclusion

A short description of the conclusion of the paper.

# 3. Abstract

Present your version of the abstract based on your understanding of the paper.

# 4. Novelty

Mention the main novelty of the paper.

# 5. Limitations and Criticism

Understand and describe the shortcomings of the technology or study discussed in the paper.

# 6. Your final thoughts

Subjectively evaluate the paper and present your views as a summary here. (Example: The relevance of the paper in your research or anything you could have done differently to realise those objectives).